# Project Documentation: 8-Bit Signed ALU

# 1. Abstract

This project involves the design, implementation, and verification of a general-purpose 8-bit Arithmetic Logic Unit (ALU) using Verilog HDL. The design targets industry-standard FPGA architectures (Xilinx Artix-7/Vivado) and supports 14 distinct operations, including signed arithmetic, bitwise logic, and barrel shifts. A key feature of this design is the robust generation of status flags (Zero, Sign, Carry, Overflow) compliant with Two's Complement signed arithmetic rules.

# 2. Technical Specifications

Hardware Description Language: Verilog (IEEE 1364-2005)

Target Device: FPGA (General Purpose)

Data Width: 8-Bit

Architecture: Combinational Logic with Behavioral Modeling

Latency: Single-Cycle Execution

# 3. Features

The ALU serves as the execution core for a simple processor, providing:

Arithmetic: ADD, SUB (with full carry/borrow logic).

Logic: AND, OR, XOR, NOT, NAND, NOR.

Shifting: Logical Left/Right, Arithmetic Right (sign-preserving), and Rotations.

Comparison: Signed 'Set Less Than' (SLT) for conditional branching.

Status Flags: Z (Zero), S (Sign), C (Carry), V (Overflow).

# 4. Port Definitions

| Port Name | Direction | Width | Description |
| --- | --- | --- | --- |
| A | Input | [7:0] | Primary Operand |
| B | Input | [7:0] | Secondary Operand |
| Opcode | Input | [3:0] | Operation Selector |
| Result | Output | [7:0] | Computed Output |
| Z | Output | 1-bit | Zero Flag |
| S | Output | 1-bit | Sign Flag |
| C | Output | 1-bit | Carry Flag |
| V | Output | 1-bit | Overflow Flag |

# 5. Instruction Set Architecture (ISA)

| Opcode | Mnemonic | Operation | Description |
| --- | --- | --- | --- |
| 0000 | ADD | A + B | Signed/Unsigned Addition |
| 0001 | SUB | A - B | Signed/Unsigned Subtraction |
| 0010 | AND | A & B | Bitwise AND |
| 0011 | OR | A ¦ B | Bitwise OR |
| 0100 | XOR | A ^ B | Bitwise XOR |
| 0101 | NOR | ~(A ¦ B) | Bitwise NOR |
| 0110 | NAND | ~(A & B) | Bitwise NAND |
| 0111 | NOT | ~A | Bitwise NOT |
| 1000 | SLT | (A < B) | Set Less Than (Signed Comparison) |
| 1001 | SLL | A << 1 | Shift Left Logical |
| 1010 | SRL | A >> 1 | Shift Right Logical |
| 1011 | SRA | A >>> 1 | Shift Right Arithmetic (Sign Extension) |
| 1100 | ROL | Rotate L | Rotate bits left (MSB to LSB) |
| 1101 | ROR | Rotate R | Rotate bits right (LSB to MSB) |

# 6. Design Implementation Details

## 6.1 Flag Logic Strategy

To prevent latch inference and ensure reliable timing, the design utilizes a 'Default-Update Strategy'. All output variables are set to logical 0 at the start of the execution block. The Z and S flags are recalculated at the very end of the block based on the final Result, ensuring they are valid for all 14 operations.

## 6.2 Overflow (V) Calculation

The Overflow flag is critical for signed arithmetic to detect when a result exceeds the 8-bit signed range (-128 to +127). It is calculated using the formula: V = ~(A[7] ^ B[7]) & (A[7] ^ Result[7]). This logic detects if two operands of the same sign produce a result of the opposite sign.

## 6.3 Arithmetic Shift Right (SRA)

Unlike a logical shift which fills with zeros, the SRA operation preserves the sign bit to maintain the mathematical validity of dividing negative numbers by 2. Implementation: Result = {A[7], A[7:1]}.

# 7. Verification Results

The design was verified using Xilinx Vivado Simulator. Key test vectors passed include:

Signed Overflow: 127 + 1 = -128 (V flag triggered High).

Zero Detection: 5 - 5 = 0 (Z flag triggered High).

Signed Comparison: -5 < 5 (Result = 1).

Shift Carry: 128 << 1 (Result = 0, C flag triggered High).

# 8. Conclusion

The 8-bit ALU successfully meets all design requirements. It provides a complete set of integer operations suitable for microcontroller datapaths and has been verified to correctly handle edge cases in signed arithmetic.